Complete all questions in 1 hour.

- Draw the logic diagram of the following gates using logsim and complete the

Truth tables.

- AND

Truth Table of AND Gate

| A | B | A.B |
| --- | --- | --- |
| 1 | 1 | 1 |
| 1 | 0 | 0 |
| 0 | 1 | 0 |
| 0 | 0 | 0 |

- NOR

Truth Table of NOR Gate

| A | B | (A+B)' |
| --- | --- | --- |
| 1 | 1 | 0 |
| 1 | 0 | 0 |
| 0 | 1 | 0 |
| 0 | 0 | 1 |

- OR

| A | B | A+B |
| --- | --- | --- |
| 1 | 1 | 1 |
| 1 | 0 | 1 |
| 0 | 1 | 1 |
| 0 | 0 | 0 |

- NAND (using NOT and AND)

Truth table of NAND Gate

| A | B | (A.B)' |
| --- | --- | --- |
| 1 | 1 | 0 |
| 1 | 0 | 1 |
| 0 | 1 | 1 |
| 0 | 0 | 1 |

- XOR using AOI

Truth Table of XOR (A.B’ + A’.B)

| A | B | A.B' + A'.B |
| --- | --- | --- |
| 1 | 1 | 0 |
| 1 | 0 | 1 |
| 0 | 1 | 1 |
| 0 | 0 | 0 |

- Use LogSim to build the equivalent circuit for the following Boolean equations.

Prove that the expressions are equivalent by computing truth table.

!A+B=!A.!B

| A | B | !(A+B) | !A . !B |
| --- | --- | --- | --- |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 |

- Draw the following circuit of half adder using LogSim.

- Draw full adder using Logsim and construct truth table.

Full Adder Truth Table

| A | B | C.in | Sum | C.out |
| --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 0 | 0 | 0 | 0 |

- Draw the logic circuit for the following Boolean equations using logsim simulator.

  - AB+C

  - A(B+C)

  - X’Y’Z’