Instruction:

Complete all questions in 2 hour.

Group: A

- Unit of computer capable of performing arithmetic, logical and data manipulation operation on binary numbers is called

  - CU

  - ALU ✓

  - I/O units

  - Processing Unit

- Arithmetic logic unit

I.   perform arithmetic operations

II.  store data

III. perform comparison

IV.  communicate with input devices

From above Correct one is.

  - I only

  - II only

  - I and II only

  - I and III only ✓

- Which of the following is component of ALU?

  - Functional Unit

  - Multiplexor

  - Instruction Decoder

  - All of the Above ✓

- Operations of Computer Arithmetic and logic unit is directed by

  - ALU itself

  - Program

  - Control Unit ✓

  - Memory Unit

- An arithmetic logic unit (ALU) is a ______________ digital electronic circuit.

  - Combinational ✓

  - Sequential

  - Both

  - None of above

- Engineering design of arithmetic logic unit determines the

  - Type and number of storing operations

  - Type and number of logical operations

  - Type and number of control operations

  - Type and number of logical and arithmetic operations ✓

- Which is the function of Decoder?

  - Perform logic and arithmetic operation

  - Selects the output we want from ALU ✓

  - Send output choice made through the decoder

  - None of the above

- Which of the following is the function of Multiplexor?

  - Perform logic and arithmetic operation

  - Selects the output we want from ALU

  - Send output choice made through the decoder ✓

  - None of the above

- Both addition and subtraction can be performed by a single circuit using ____________

  - Multiplexor

  - Controlled Inversion ✓

  - Half Adder

  - Fuller Adder

Group B

- Design a combinational Logic circuit that selects and generates any of the following logic and arithmetic functions listed below.

A XOR B

A NOR B

A + B

A – B

- Design a digital circuit that performs the four logical operations of exclusive-OR, NOR, NAND and OR. Use two selection variables. Show the logic diagram of one typical stage. Discuss the working mechanism of the circuit that you have constructed.

- Following diagram shows a 4 bit adder/subtractor.  Design the circuit diagram using Logsim. Discuss how the circuit performs addition and subtraction.